| REGISTRATION NUMBER | : | EN214/3911/2024 |
| --- | --- | --- |
| NAME | : | REAGAN SIMIYU |
| INSTITUTION | : | MURANGʼA UNIVERSITY OF TECHNOLOGY |
| SCHOOL | : | ENGINEERING AND TECHNOLOGY |
| DEPARTMENT | : | MECHANICAL ENGINEERING |
| COURSE NAME | : | THERMODYNAMICS |
| TITLE | : | PERFORMANCE TEST OF A BOILER |
| EXPERIMENT DATE | : | 06/05/2025 |
| SUBMISSION DATE | : | 25/05/2025 |

INTRODUCTION

A steam boiler is a closed pressure vessel used to convert water into steam by applying heat energy from fuel combustion.

The performance of a boiler is determined by its ability to generate steam efficiently while maintaining safe operating pressure and minimum fuel consumption. Boiler efficiency is a measure of how effectively the heat from fuel is converted into useful steam energy.

This experiment involved operating a Class B6 boiler and monitoring parameters such as steam generation rate, steam pressure, fuel consumption, and water consumption under different operating conditions.

OBJECTIVE

To operate a boiler and find quantity of steam generated, steam pressure and boiler efficiency and also gain practical experience and basic practice in boiler operation.

THEORY

When fuel burns in the boiler furnace, heat energy is released and transferred to water inside the boiler tubes or shell. The water absorbs the heat and changes into steam.

The efficiency of a boiler is defined as the ratio of useful heat utilized in steam generation to the total heat supplied by the fuel.

The quantity of steam generated depends on: fuel combustion rate, heat transfer efficiency, water supply rate and boiler pressure.

APPARATUS

•  Class B6 steam boiler

•  Fuel flow meter

•  Water flow meter

•  Steam Delivery Valve (SDV)

•  Pressure gauge

•  Thermometer

•  Water tank

•  Stopwatch

•  Steam pipes and fittings

PROCEDURE

1..The boiler was inspected before operation to ensure all valves, gauges, and safety devices were functioning properly.

2..Water was supplied to the boiler until the correct water level was reached.

3..The fuel supply was opened and the burner ignited. The boiler was allowed to heat until steam generation began.

4..The Steam Delivery Valve (SDV) was adjusted to regulate steam flow at different stages.

5..Boiler pressure, fuel flow rate, and water flow rate were recorded at intervals of two minutes for ten minutes.

6..Initial and final readings of fuel and water flow meters were recorded.

7..Saturated steam conditions were maintained at the boiler outlet throughout the experiment.

8..The quantity of steam generated and boiler efficiency were calculated using the recorded data.

RESULTS

| Time (min) | P1 | T3 | T1 | Boiler Water l/10min | Boiler Fuel l/10min | ΔH | SΔv |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3.50 | 23.6 | 118.3 | 583.9 | 7535.6 | 55 | 1/3 |
| 2 | 3.40 | 23.6 | 118.4 |  |  |  |  |
| 4 | 3.25 | 23.6 | 119.2 |  |  |  |  |
| 6 | 3.25 | 23.6 | 119.1 |  |  |  |  |
| 8 | 3.25 | 23.6 | 119.1 |  |  |  |  |
| 10 | 3.2 | 23.6 | 119.7 | 585.9 | 7537.1 | 35 | 2/3 |
| 20 |  |  |  | 609.1 | 7546.4 | 5 | 3/3 |
| 30 |  |  |  | 619.4 | 7557.1 |  |  |

ANALYSIS

Efficiency = (ms × (H − Hf)) / (mf × CV) × 100%

Where:

Ms = mass of steam generated

H = enthalpy of steam at boiler pressure and temperature

Hf = enthalpy of feed water

Mf = mass of fuel consumed

CV = calorific value of fuel (44,800 kJ/kg)

619.4 − 583.9 = 35.5 L = 35.5 kg

7537.1 − 7535.6 = 1.5 × 3 = 4.5 L

Mf = volume × density = 0.85 kg/L

0.85 × 4.5 = 3.825 kg

Average T1 = 125°C

Average T3 = 23.6°C

Hf = 23.6 × 4.187 = 98.8 kJ/kg

Efficiency = ((35.5 × (2715 − 98.8)) / (3.825 × 44800)) × 100% = 54.2%

DISCUSSION

The experiment successfully demonstrated the operation and performance evaluation of a Class B6 steam boiler. Steam generation occurred when sufficient heat was transferred from fuel combustion to the water inside the boiler.

The boiler pressure increased gradually as steam accumulated in the boiler drum. The Steam Delivery Valve controlled the steam flow rate and affected the operating pressure.

Several factors influenced the boiler performance: combustion efficiency, heat transfer rate, steam leakage, fuel quality, and boiler insulation.

Heat losses through flue gases, radiation, and incomplete combustion reduced the overall efficiency of the boiler.

Sources of error may be due to: heat loss to the surroundings, inaccurate flow meter readings, steam leakage from pipes and fittings, pressure gauge reading errors, and incomplete combustion of fuel.

Precautions to be taken are: ensuring the correct boiler water level is maintained, avoiding operating the boiler above safe pressure limits, ensuring all steam pipes and valves are leak-free, recording readings accurately at specified intervals, and wearing protective equipment during operation.

CONCLUSIONS

The experiment was successfully conducted to operate a Class B6 steam boiler and determine its performance characteristics. The quantity of steam generated, steam pressure, and boiler efficiency were determined successfully.

The experiment showed that steam generation and pressure increase with increased fuel combustion and heat transfer. Boiler efficiency depended on combustion quality and operating conditions.

The practical session provided valuable experience in boiler operation, monitoring, safety procedures, and performance evaluation.

REFERENCES

1. R.K. Rajput, Thermal Engineering.

2. R.S. Khurmi and J.K. Gupta, Thermal Engineering.

3. P.K. Nag, Engineering Thermodynamics.

4. C.P. Arora, Thermodynamics.